CAMSHAFTS for the Renault 5

| Part No. | Application | Power Band | Power Increase | Duration Inl Exh |  | Valve Lift Inl Exh |  | Timing Inl Open BTDC Close ABDC Exh Open BBDC Close ATDC |  | Full Lift Inl ATDC Exh BTDC |  | Valve Clearance | Price ex VAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Not Turbo, just C1J | Renault C1J per blue Haynes manual, at checking clearance per dialogys of 0.30mm inlet and 0.35mm exhaust |  |  | 248° | 248° |  |  | Inl 12-56 Exh 56-12 N.B. this is duration at checking clearance 0.30mm in/ 0.35 ex | Overlap 24° | 112° | 112° |  |  |
| Piper cam, Not Turbo, R5BP320 | Race for 5 / 9 / 11 from very old data sheet | 4000- 8000 |  | 328° | 328° | .415" 10.54mm | .415" 10.54mm | Inl 60-88 Exh 88-60 | Overlap 120° | 104° | 104° | .017" / .43mm .017" / .43mm |  |
| C1J-82-88-70 | Renault 5GTT from the Cup manual and also Paul B’s dialogys for the standard 5GTT. |  |  | 260° | 260° | 7.3mm to 7.5mm |  | Inl 14-66 Exh 54-26 N.B. this is duration at checking clearance 0.30mm in/ 0.35 ex | Overlap 40° | 116° | 104° | Cold setting .008" / .20mm .010" / .25mm |  |
| 840-26-C6J-28-50 | Renault 5 Gordini Turbo with X flow hemi head |  |  | 244° | 244° |  |  | Inl 10-54 Exh 54-10 N.B. this is duration at checking clearance 0.40mm in/ 0.40 ex | Overlap 20° | 112° | 112° |  |  |
| 840-30-C7K-00 | Renault 5 Turbo 1980 with X flow hemi head |  |  | 260° | 260° |  |  | Inl 28-52 Exh 66-14 N.B. this is duration at checking clearance 0.50mm in/ 0.50 ex | Overlap 42° | 102° | 116° | .012" / .30mm .016" / .40mm |  |
|  | K-Tec |  |  | 260° | 260° | .322" 8.433mm | .330" 8.382 mm | Inl 20-60 Exh 60-20 | Overlap 40° | 110° | 110° | .015" / .38mm .017" / .43mm |  |

PIPER CAMS Valve lifts quoted assume a rocker ratio of 1.52:1

| R5TBP270 | FAST ROAD from very old data sheet | 2000- 6000 |  | 270° | 270° | .320" 8.13mm | .320" 8.13mm | Inl 23-67 Exh 67-23 | 46° | 112° | 112° | .014" / .35mm .016" / .40mm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R5TBP270 No. 2 | FAST ROAD | 2000- 6000 | 15 BHP | 260° | 260° | .342" 8.68mm | .342" 8.68mm | Inl 20-60 Exh 60-20 | 40° | 110° | 110° | .010" / .25mm .012" / .30mm |  |
| R5TBP270 | FAST ROAD |  |  | 260° | 260° |  |  | Inl 21-59 Exh 59-21 | 42° | 109° | 109° |  |  |
| R5TBP285 @110 | ULTIMATE ROAD BP285@110 REPROFILED CAM |  |  |  |  |  |  |  |  | 110° | 110° |  |  |
| R5TBP285 No. 1 | ROAD / RALLY from very old data sheet | 2200- 6500 |  | 270° | 270° | .375" 9.53mm | .375" 9.53mm | Inl 23-67 Exh 67-23 | 46° | 112° | 112° | .010" / .25mm .012" / .30mm |  |
| R5TBP285 | ROAD RALLY Piper valve springs Part no. VSSR5 must be used |  |  |  |  | 9.1mm |  |  |  |  |  |  |  |
| R5TBP285 | ROAD RALLY Piper valve springs Part no. VSSR5 must be used |  |  | 270° | 270° | 9.53mm |  | Inl 26-62 Exh 62-26 | 52° | 109° | 109° |  |  |
| R5TBP285 No. 4 | ROAD RALLY Piper valve springs Part no. VSSR5 must be used | 2500- 6500 | 20 BHP | 270° | 270° | .394" 10.01mm | .394" 10.01mm | Inl 23-67 Exh 67-23 | 46° | 112° | 112° | .008" / .20mm .010" / .25mm |  |
| R5TBP292 105/465/112 | GDI |  |  | 280° | 260° | 10.008 | 9.906 | Inl 28-72 Exh 62-18 | 46° | 112° | 112° | .008" / .20mm .010" / .25mm |  |
| R5TGRP A | Group A from very old data sheet | 2500- 7000 |  | 276° | 276° | .348" 8.84mm | .348" 8.84mm | Inl 26-70 Exh 70-26 | 52° | 112° | 112° | .010" / .25mm .012" / .30mm |  |
| R5TBP300 | RALLY (Group A) | 2500- 6500 | 20 BHP | 276° | 276° | .360" 9.14mm | .360" 9.14mm | Inl 26-70 Exh 70-26 | 52° | 112° | 112° | .008" / .20mm .010" / .25mm |  |
| R5TBP300 | RALLY (Group A) |  |  | 276° | 276° | 9.53mm | 9.53mm | Inl 28-68 Exh 68-28 | 56° | 110° | 110° |  |  |
| R5TBP300 | RALLY (Group A) |  |  | 280° | 280° | 9.18mm | 9.18mm | Inl 30-70 Exh 70-30 | 60° | 110° | 110° |  |  |
| R5TBP300 | RALLY (Group A) |  |  | 284° | 284° | 8.84mm | 8.84mm | Inl 36-68 Exh 68-36 | 72° | 106° | 106° |  |  |
| R5TBP1011@108 | Sparkie special No.1 Retarded cam by 4° |  | 30 BHP? | 276° | 276° | 9.80mm | 9.80mm | Inl 30-66 Exh 66-30 | 60° | 108° | 108° |  |  |
| R5TBP390@114 | Sparkie special No. 2 |  |  | 280° | 280° | 10.79mm | 10.79mm | Inl 26-74 Exh 74-26 | 52° | 114° | 114° |  |  |

| Part No. | Application | Power Band | Power Increase | Inl Exh Duration | Inl Exh Valve Lift | Inl Open BTDC Close ABDC Exh Open BBDC Close ATDC Timing | Inl ATDC Exh BTDC Full Lift | Valve Clearance | Price ex VAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

KENT CAMS  Cam lift stated as 6.17mm for RT150 and 6.83mm for RT200.  Mechanical profiles checked at 0.016"/0.40mm checking height, ie duration at 16 thou valve clearance.  RT150 ostensibly equals 150bhp, RT200 for 200bhp engine.  Designed in France many years ago.  RT150 for a standard engine.  RT200 for uprated one.

| R5RT150 | SPORTS | 2000- 6500 |  | 252 | 252 | 8.5mm | 8.45mm | Inl 14-58 Exh 58-14 N.B. this is duration at checking clearance 0.40mm in/ 0.40 ex | Overlap 28° | 112° | 112° | .010" / .25mm .012" / .30mm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R5RT200 older version | SUPERSPORTS | 3000- 7000 |  | 264 | 264 | 9.42mm | 9.37mm | Inl 26-58 Exh 58-26 N.B. this is duration at checking clearance 0.40mm in/ 0.40 ex | Overlap 52° | 106° | 106° | .010" / .25mm .012" / .30mm |  |
| R5RT200 newer | SUPERSPORTS | 3000- 7000 |  | 264 | 264 | 9.42mm | 9.37mm | Inl 20-64 Exh 64-20 N.B. this is duration at checking clearance 0.40mm in/ 0.40 ex | Overlap 40° | 112° | 112° | .010" / .25mm .012" / .30mm |  |
| Part No. | Application | Power Band | Power Increase | Inl Exh Duration |  | Inl Exh Valve Lift |  | Inl Open BTDC Close ABDC Exh Open BBDC Close ATDC Timing |  | Inl ATDC Exh BTDC Full Lift |  | Valve Clearance | Price ex VAT |

Additional Components from Piper: PULSR5 - Renault 5 turbo vernier pulley £91 VSSR5 - Renault 5 turbo single valve springs £38 Kent Cams also do a vernier pulley.

Additional Information:

Piper 01233 500 200

www.kentcams.com 01303 248666

N.B.  Kent Valve lift and duration are not quite so black and white as the table suggests.

For the Kent RT200

cam lift 6.83mm

rocker ratio 1:1.52:

minus suggested inlet clearance of 0.25mm

valve lift is 10.13mm

but Kent state inlet valve lift as 9.42mm

I guess if it's 9.42mm at their stated checking height of 0.40mm, then at the suggested clearance height of 0.25mm it will be 0.15mm more?

So 9.57mm and for the exhaust valve 9.47mm.

Can anyone explain this error?

And can anyone guess, I mean work out, what the duration will be??

Kent v Piper: A well respected and knowledgeable RTOC member has measured both the Kent RT200 and Piper BP285 and determined that the Kent has the longer duration, whatever the spec sheets say.  It also has a faster rate of lift meaning it has opened the valve further for the same amount or degrees of rotation.

It’s quite popular to advance a cam by maybe 4°.  This tends to have the effect of making the car more torquey and less revvy.  Conversely, when retarding a cam by maybe 4° the effect is reversed and they engine wants to rev higher but feels rather lacking in mid revs power.  Piper said to me it’s best to dial their cams in to what they specify.  It seems to be a god idea to use a vernier pulley as they vary by quite a few degrees when put on with the standard pulley!  As much a 9° out!!!

See this RTOC thread:

http://www.rtoc.org/boards/thread.asp?TopicID=60934  Re: Andrew Cookes cam measurements May to Jul 2007.

http://www.rtoc.org/boards/thread.asp?TopicID=61500  Re: Andrew Cookes cam measurements Jun 2007.

http://www.rtoc.org/boards/thread.asp?ShowType=1&SortColumn=1&SortOrder=2&TimePeriod=5&BoardID=2&TopicID=26374&page=4

http://www.rtoc.org/boards/thread.asp?TopicID=43600

http://www.rtoc.org/boards/thread.asp?TopicID=34125  thread regarding the cup car cam.

http://www.rtoc.org/boards/thread.asp?TopicID=61662  Valve springs.

Note:

Piper and Kent data from their www’s and data sheets plus my phone call to Kent Tech.

GDI cam details were published on the GTTurbo owners club boards.

Sparkie cam info courtesy of Sparkie.

K-Tech cam info from their product data sheet.

What’s the difference between a repro, a blank, a billet?

A re-profile is your old cam re-ground.

A blank is ground from a new casting.  A new casting has oversized un-profiled areas for the lobes, just like Renault would have used from new.

A billet is machined out of a steel rod.

lobe on the left is exhaust, one on the right is inlet

For more cam graphs see:   http://www.rtoc.org/library/default.asp?id=3358